程式：SetTime_fromNet 網路校時

開啟程式SetTime_fromNet

程式位址：https://github.com/brucetsao/BruceCourses/blob/master/105ANQU_IOT/Code/SetTime_fromNet/SetTime_fromNet.ino

#include <Wire.h> #include "RTClib.h" RTC_DS1307 RTC; // above is used for RTC #include "PMType.h" #include <WiFi.h> #include <PubSubClient.h> #include <WiFiUdp.h> // Aboev is used for WIFI uint8_t MacData[6]; char ssid[] = "TSAO_1F"; // your network SSID (name) char pass[] = "TSAO1234"; // your network password char server[] = "gpssensor.ddns.net"; // the MQTT server of LASS #define MAX_CLIENT_ID_LEN 10 #define MAX_TOPIC_LEN 50 char clientId[MAX_CLIENT_ID_LEN]; char outTopic[MAX_TOPIC_LEN]; IPAddress Meip , Megateway , Mesubnet ; String MacAddress ; int status = WL_IDLE_STATUS; WiFiUDP Udp; const char ntpServer[] = "pool.ntp.org"; const long timeZoneOffset = 28800L; const int NTP_PACKET_SIZE = 48; // NTP time stamp is in the first 48 bytes of the message const byte nptSendPacket[ NTP_PACKET_SIZE] = { 0xE3, 0x00, 0x06, 0xEC, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x31, 0x4E, 0x31, 0x34, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00 }; byte ntpRecvBuffer[ NTP_PACKET_SIZE ]; #define LEAP_YEAR(Y) ( ((1970+Y)>0) && !((1970+Y)%4) && ( ((1970+Y)%100) ¦¦ !((1970+Y)%400) ) ) static const uint8_t monthDays[] = {31, 28, 31, 30, 31, 30, 31, 31, 30, 31, 30, 31}; // API starts months from 1, this array starts from 0 uint32_t epochSystem = 0; // timestamp of system boot up unsigned long epoch ; int NDPyear, NDPmonth, NDPday, NDPhour, NDPminute, NDPsecond; // this is used for WIFI and NTP void setup() { Serial.begin(9600); initRTC() ; // init RTC Modules MacAddress = GetWifiMac() ; ShowMac() ; initializeWiFi(); ShowNTPDateTime() ; SetRTCTime(NDPyear, NDPmonth, NDPday, NDPhour, NDPminute, NDPsecond); delay(1500); } void loop() { // run over and over delay(1000); // delay 1 minute for next measurement Serial.print("Now RTC Data and Time is :") ; Serial.print(ShowDateTime()) ; Serial.print("\n") ; delay(1000) ; } void ShowNTPDateTime() { retrieveNtpTime() ; getCurrentTime(epoch + timeZoneOffset, &NDPyear, &NDPmonth, &NDPday, &NDPhour, &NDPminute, &NDPsecond); //ttt->year = NDPyear ; Serial.print("NDP Date is :"); Serial.print(StringDate(NDPyear, NDPmonth, NDPday)); Serial.print("and "); Serial.print("NDP Time is :"); Serial.print(StringTime(NDPhour, NDPminute, NDPsecond)); Serial.print("\n"); } void ShowMac() { Serial.print("MAC:"); Serial.print(MacAddress); Serial.print("\n"); } void ShowInternetStatus() { if (WiFi.status()) { Meip = WiFi.localIP(); Serial.print("Get IP is:"); Serial.print(Meip); Serial.print("\n"); } else { Serial.print("DisConnected:"); Serial.print("\n"); } } void initRTC() { Wire.begin(); RTC.begin(); if (! RTC.isrunning()) { Serial.println("RTC is NOT running!"); } } String ShowDateTime() { return StrDate() + " " + StrTime() ; } String StrDate() { String ttt ; //nowT = now; DateTime now = RTC.now(); ttt = print4digits(now.year()) + "-" + print2digits(now.month()) + "-" + print2digits(now.day()) ; //ttt = print4digits(NDPyear) + "/" + print2digits(NDPmonth) + "/" + print2digits(NDPday) ; return ttt ; } String StringDate(int yyy, int mmm, int ddd) { String ttt ; //nowT = now; ttt = print4digits(yyy) + "-" + print2digits(mmm) + "-" + print2digits(ddd) ; return ttt ; } String StrTime() { String ttt ; // nowT = RTC.now(); DateTime now = RTC.now(); ttt = print2digits(now.hour()) + ":" + print2digits(now.minute()) + ":" + print2digits(now.second()) ; // ttt = print2digits(NDPhour) + ":" + print2digits(NDPminute) + ":" + print2digits(NDPsecond) ; return ttt ; } String StringTime(int hhh, int mmm, int sss) { String ttt ; ttt = print2digits(hhh) + ":" + print2digits(mmm) + ":" + print2digits(sss) ; return ttt ; } String print2digits(int number) { String ttt ; if (number >= 0 && number < 10) { ttt = String("0") + String(number); } else { ttt = String(number); } return ttt ; } String print4digits(int number) { String ttt ; ttt = String(number); return ttt ; } String GetWifiMac() { String tt ; String t1, t2, t3, t4, t5, t6 ; WiFi.status(); //this method must be used for get MAC WiFi.macAddress(MacData); Serial.print("Mac:"); Serial.print(MacData[0], HEX) ; Serial.print("/"); Serial.print(MacData[1], HEX) ; Serial.print("/"); Serial.print(MacData[2], HEX) ; Serial.print("/"); Serial.print(MacData[3], HEX) ; Serial.print("/"); Serial.print(MacData[4], HEX) ; Serial.print("/"); Serial.print(MacData[5], HEX) ; Serial.print("~"); t1 = print2HEX((int)MacData[0]); t2 = print2HEX((int)MacData[1]); t3 = print2HEX((int)MacData[2]); t4 = print2HEX((int)MacData[3]); t5 = print2HEX((int)MacData[4]); t6 = print2HEX((int)MacData[5]); tt = (t1 + t2 + t3 + t4 + t5 + t6) ; Serial.print(tt); Serial.print("\n"); return tt ; } String print2HEX(int number) { String ttt ; if (number >= 0 && number < 16) { ttt = String("0") + String(number, HEX); } else { ttt = String(number, HEX); } return ttt ; } // send an NTP request to the time server at the given address void retrieveNtpTime() { Serial.println("Send NTP packet"); Udp.beginPacket(ntpServer, 123); //NTP requests are to port 123 Udp.write(nptSendPacket, NTP_PACKET_SIZE); Udp.endPacket(); if (Udp.parsePacket()) { Serial.println("NTP packet received"); Udp.read(ntpRecvBuffer, NTP_PACKET_SIZE); // read the packet into the buffer unsigned long highWord = word(ntpRecvBuffer[40], ntpRecvBuffer[41]); unsigned long lowWord = word(ntpRecvBuffer[42], ntpRecvBuffer[43]); unsigned long secsSince1900 = highWord << 16 ¦ lowWord; const unsigned long seventyYears = 2208988800UL; // epoch = secsSince1900 - seventyYears + timeZoneOffset ; epoch = secsSince1900 - seventyYears ; epochSystem = epoch - millis() / 1000; } } void SetRTCTime( int yr, int mon, int dd, int hr, int mins, int secs) { RTC.adjust(DateTime(yr, mon, dd, hr, mins, secs)); } void getCurrentTime(unsigned long epoch, int *year, int *month, int *day, int *hour, int *minute, int *second) { int tempDay = 0; *hour = (epoch % 86400L) / 3600; *minute = (epoch % 3600) / 60; *second = epoch % 60; *year = 1970; *month = 0; *day = epoch / 86400; for (*year = 1970; ; (*year)++) { if (tempDay + (LEAP_YEAR(*year) ? 366 : 365) > *day) { break; } else { tempDay += (LEAP_YEAR(*year) ? 366 : 365); } } tempDay = *day - tempDay; // the days left in a year for ((*month) = 0; (*month) < 12; (*month)++) { if ((*month) == 1) { if (LEAP_YEAR(*year)) { if (tempDay - 29 < 0) { break; } else { tempDay -= 29; } } else { if (tempDay - 28 < 0) { break; } else { tempDay -= 28; } } } else { if (tempDay - monthDays[(*month)] < 0) { break; } else { tempDay -= monthDays[(*month)]; } } } (*month)++; *day = tempDay + 2; // one for base 1, one for current day } void initializeWiFi() { while (status != WL_CONNECTED) { Serial.print("Attempting to connect to SSID: "); Serial.println(ssid); // Connect to WPA/WPA2 network. Change this line if using open or WEP network: status = WiFi.begin(ssid, pass); //status = WiFi.begin(ssid); // wait 10 seconds for connection: delay(10000); } Serial.print("Success to connect AP:") ; Serial.print(ssid) ; Serial.print("\n") ; // local port to listen for UDP packets Udp.begin(2390); } void printWifiData() { // print your WiFi shield's IP address: Meip = WiFi.localIP(); Serial.print("IP Address: "); Serial.println(Meip); Serial.print("\n"); // print your MAC address: byte mac[6]; WiFi.macAddress(mac); Serial.print("MAC address: "); Serial.print(mac[5], HEX); Serial.print(":"); Serial.print(mac[4], HEX); Serial.print(":"); Serial.print(mac[3], HEX); Serial.print(":"); Serial.print(mac[2], HEX); Serial.print(":"); Serial.print(mac[1], HEX); Serial.print(":"); Serial.println(mac[0], HEX); // print your subnet mask: Mesubnet = WiFi.subnetMask(); Serial.print("NetMask: "); Serial.println(Mesubnet); // print your gateway address: Megateway = WiFi.gatewayIP(); Serial.print("Gateway: "); Serial.println(Megateway); }

SetTime_fromNet 程式重點解說

- #include <WiFi.h>  使用網路必要函數

- uint8_t MacData[6];   儲存 MAC資料

- initRTC() ;  初始化時鐘模組

- ShowNTPDateTime() ;   取得網路時間並顯示

- retrieveNtpTime() ;取得網路時間

- getCurrentTime(epoch+timeZoneOffset, &NDPyear, &NDPmonth, &NDPday, &NDPhour, &NDPminute, &NDPsecond); 將網路時間存入變數

- SetRTCTime(NDPyear, NDPmonth, NDPday, NDPhour, NDPminute, NDPsecond);  將取得網路時間調整時鐘模組