程式：APMODEControlRelay 使用AP模式建立網頁伺服器

開啟程式APMODEControlRelay

程式位址：https://github.com/brucetsao/BruceCourses/blob/master/105ANQU_IOT/Code/APMODEControlRelay/APMODEControlRelay.ino

#include <WiFi.h> #define RelayPin 8 #define Turnon HIGH #define Turnoff LOW char ssid[] = "Ameba"; //Set the AP's SSID char pass[] = "12345678"; //Set the AP's password char channel[] = "1"; //Set the AP's channel int status = WL_IDLE_STATUS; // the Wifi radio's status int keyIndex = 0; // your network key Index number (needed only for WEP) WiFiServer server(80); void setup() { //Initialize serial and wait for port to open: Serial.begin(9600); pinMode(RelayPin, OUTPUT); // set the LED pin mode // check for the presence of the shield: if (WiFi.status() == WL_NO_SHIELD) { Serial.println("WiFi shield not present"); while (true); } String fv = WiFi.firmwareVersion(); if (fv != "1.1.0") { Serial.println("Please upgrade the firmware"); } // attempt to start AP: while (status != WL_CONNECTED) { Serial.print("Attempting to start AP with SSID: "); Serial.println(ssid); status = WiFi.apbegin(ssid, pass, channel); delay(10000); } //AP MODE already started: Serial.println("AP mode already started"); Serial.println(); server.begin(); printWifiData(); printCurrentNet(); } void loop() { WiFiClient client = server.available(); // listen for incoming clients if (client) { // if you get a client, Serial.println("new client"); // print a message out the serial port String currentLine = ""; // make a String to hold incoming data from the client while (client.connected()) { // loop while the client's connected if (client.available()) { // if there's bytes to read from the client, char c = client.read(); // read a byte, then Serial.write(c); // print it out the serial monitor if (c == '\n') { // if the byte is a newline character // if the current line is blank, you got two newline characters in a row. // that's the end of the client HTTP request, so send a response: if (currentLine.length() == 0) { // HTTP headers always start with a response code (e.g. HTTP/1.1 200 OK) // and a content-type so the client knows what's coming, then a blank line: client.println("HTTP/1.1 200 OK"); client.println("Content-type:text/html"); client.println(); client.print("<title>Ameba AP Mode Control Relay</title>"); client.println(); client.print("<html>"); client.println(); // the content of the HTTP response follows the header: client.print("Click <a href=\"/H\">here</a> turn Power on<br>"); client.print("Click <a href=\"/L\">here</a> turn Power off<br>"); client.println(); client.print("</html>"); client.println(); // The HTTP response ends with another blank line: client.println(); // break out of the while loop: break; } else { // if you got a newline, then clear currentLine: currentLine = ""; } } else if (c != '\r') { // if you got anything else but a carriage return character, currentLine += c; // add it to the end of the currentLine } // Check to see if the client request was "GET /H" or "GET /L": if (currentLine.endsWith("GET /H")) { digitalWrite(RelayPin, Turnon); // GET /H turns the LED on } if (currentLine.endsWith("GET /L")) { digitalWrite(RelayPin, Turnoff); // GET /L turns the LED off } } } // close the connection: client.stop(); Serial.println("client disonnected"); } } void printWifiData() { // print your WiFi shield's IP address: IPAddress ip = WiFi.localIP(); Serial.print("IP Address: "); Serial.println(ip); // print your subnet mask: IPAddress subnet = WiFi.subnetMask(); Serial.print("NetMask: "); Serial.println(subnet); // print your gateway address: IPAddress gateway = WiFi.gatewayIP(); Serial.print("Gateway: "); Serial.println(gateway); Serial.println(); } void printCurrentNet() { // print the SSID of the AP: Serial.print("SSID: "); Serial.println(WiFi.SSID()); // print the MAC address of AP: byte bssid[6]; WiFi.BSSID(bssid); Serial.print("BSSID: "); Serial.print(bssid[0], HEX); Serial.print(":"); Serial.print(bssid[1], HEX); Serial.print(":"); Serial.print(bssid[2], HEX); Serial.print(":"); Serial.print(bssid[3], HEX); Serial.print(":"); Serial.print(bssid[4], HEX); Serial.print(":"); Serial.println(bssid[5], HEX); // print the encryption type: byte encryption = WiFi.encryptionType(); Serial.print("Encryption Type:"); Serial.println(encryption, HEX); Serial.println(); }

APMODEControlRelay程式重點解說

- #include <WiFi.h>  使用網路必要函數

- WiFiServer server(80);   啟動PORT 80進行

- status = WiFi.begin(ssid);		不使用加密連AP

- status = WiFi.begin(ssid, pass);	使用加密連AP

- server.begin(); 	開始啟動PORT 80 傾聽

- printWifiStatus();	列印網路資訊

- WiFiClient client = server.available(); 有人連接Port 80

- client.connected()  有用戶連接中

- client.available()	用戶送資料進來

- char c = client.read();	讀出用戶送的資料(一個位元組)

- client.println(“HTTP/1.1 200 OK”); ….  送給用戶端一段HTML碼，用瀏覽器方能顯示

- currentLine.endsWith(“GET /H”)  判斷是否用/H 結束

- currentLine.endsWith(“GET /L”)  判斷是否用/L 結束

- client.stop();	與用戶連線停止傳輸資料