程式：Set_RTC_Data (設定RTC時間)

開啟程式Set_RTC_Data

程式位址：https://github.com/brucetsao/BruceCourses/blob/master/105ANQU_IOT/Code/Set_RTC_Data/Set_RTC_Data.ino

#include <Wire.h> #include "RTClib.h" RTC_DS1307 RTC; int mYear = 2016 ; int mMonth = 1 ; int mDay = 1 ; int mHour = 12 ; int mMinute = 12 ; int mSecond = 12 ; void setup() { Serial.begin(9600); initRTC() ; RTC.adjust(DateTime(mYear,mMonth,mDay,mHour,mMinute,mSecond)); } void loop() { Serial.print("Now Data and Time is :") ; Serial.print(ShowDateTime()) ; Serial.print("\n") ; delay(1000) ; } void initRTC() { Wire.begin(); RTC.begin(); if (! RTC.isrunning()) { Serial.println("RTC is NOT running!"); } } String ShowDateTime() { return StrDate() + " " +StrTime() ; } String StrDate() { String ttt ; //nowT = now; DateTime now = RTC.now(); ttt = print4digits(now.year()) + "-" + print2digits(now.month()) + "-" + print2digits(now.day()) ; //ttt = print4digits(NDPyear) + "/" + print2digits(NDPmonth) + "/" + print2digits(NDPday) ; return ttt ; } String StringDate(int yyy,int mmm,int ddd) { String ttt ; //nowT = now; ttt = print4digits(yyy) + "-" + print2digits(mmm) + "-" + print2digits(ddd) ; return ttt ; } String StrTime() { String ttt ; // nowT = RTC.now(); DateTime now = RTC.now(); ttt = print2digits(now.hour()) + ":" + print2digits(now.minute()) + ":" + print2digits(now.second()) ; // ttt = print2digits(NDPhour) + ":" + print2digits(NDPminute) + ":" + print2digits(NDPsecond) ; return ttt ; } String StringTime(int hhh,int mmm,int sss) { String ttt ; ttt = print2digits(hhh) + ":" + print2digits(mmm) + ":" + print2digits(sss) ; return ttt ; } String print2digits(int number) { String ttt ; if (number >= 0 && number < 10) { ttt = String("0") + String(number); } else { ttt = String(number); } return ttt ; } String print4digits(int number) { String ttt ; ttt = String(number); return ttt ; }

Set_RTC_Data程式重點解說

- #include <Wire.h>   	I2C通訊用

- #include “RTClib.h“	DS1307 時鐘模組用

- RTC_DS1307 RTC;	宣告時鐘物件

- initRTC() ;	啟動時鐘物件(自訂)

- Wire.begin();   開始I2C通訊

- RTC.begin();	開始時鐘物件通訊

- RTC.isrunning()   時鐘物件順利啟動

- RTC.adjust(DateTime(mYear,mMonth,mDay,mHour,mMinute,mSecond));   調整設定DS1307 時鐘模組時間內容